HW3 Yukai Jin yj25b

| Test file | Forwarding disabled | EX/MEM -> EX Enabled (f 1) | EX/MEM & MEM/WB -> EX enabled (f 2) |
| --- | --- | --- | --- |
| Instruction.txt | 18 cycles | 14 | 12 |
| Instruction2.txt | 23 cycles | 19 | 16 |
| Instruction3.txt | 16 cycles | 13 | 11 |

Explanation:

We only need to take care of ALU (ADD, SUB, DEV, MUL) and LW, only theses instructions have RAW hazard. When implementing, as we have return False (no Hazard) to be the final situation (at the end of hasDependency() function), we need to consider what situations trigger the Hazard fault.

- EX/MEM-> EX Enabled (f 1)

In the f = 1 case, a forwarding path exists between the EX and MEM stages.
This allows results from ALU operations in the MEM stage to be bypassed directly to the next instruction in EX, avoiding unnecessary stalls.

However, there is an exception when the instruction currently in the Decode (ID) stage depends on two different previous ALU results.
In this case, one of the needed operands might only be available at the WB stage, even though the other operand can already be forwarded from the MEM stage.
Because the EX/MEM forwarding path cannot supply the WB-stage value yet, the dependent instruction must wait one bubble cycle until that WB result becomes available.

Additionally, when the decoding instruction depends on a load (LW) instruction, the load’s data will only be ready after the MEM stage, meaning it must wait two cycles before execution can proceed.

Therefore, in the implementation, two separate if conditions are required:

- one to handle the ALU result still in the WB stage(no matter what in MEM), and

- one to handle the LW result in either the MEM or WB stage.

- EX/MEM & MEM/WB -> EX enabled (f 2)

In the f = 2 case, forwarding paths exist from both MEM and WB to the EX stage.
This provides full bypass coverage for all ALU operations, since results from either stage can be forwarded to the dependent instruction in EX without delay.

As a result, the only remaining stall condition occurs when the dependent instruction needs data from a load (LW) instruction that is currently in the MEM stage, because the data from memory has not yet been fetched and cannot be forwarded.

Consequently, only one if condition is necessary in this configuration:

- to handle the single LW-in-MEM hazard.